User’s Manual

Version 2.5

November  1997

# Liability, Warranty and Trademark

I, the Author, make no warranty of any kind, expressed or implied, including but not limited to any warranties of fitness for a particular purpose.  In no event shall the authors be liable for any incidental or consequential damage arising from the use of, or inability to use, these programs. I hereby deny any liability to the maximum extent permitted by law.

You are fully responsible for everything you are doing with these programs!

Power Render and WordUp Graphics Toolkit are trademarks of Egerter Software.

3D Studio is a trademark of Autodesk Inc.

All other trademarks are the property of their respective owners.

Copyright  1997 Egerter Software

# Index

Liability, Warranty and Trademark	2

Index	3

1.0 Introduction	5

1.1 System Requirements	5

1.2 Installing Power Render	6

1.3 Overview	6

1.31 General Comments	6

1.32 Power Render Terminology	6

1.4 Coordinate Systems	8

1.41 Object Space (3D)	8

1.42 World Space (3D)	8

1.43 Camera Space (3D)	9

1.44 Viewport Space (2D)	9

1.45 Device Space (2D)	9

2.0 Initializing Power Render	10

3.0 Device Detection and Initialization	11

4.0 Reserving Space for Rendering Data	13

5.0 Setting the Video Mode	14

6.0 Viewports	15

7.0 Loading and Building Objects	16

7.1 Importing from 3D Studio	16

7.1.1 Keyframing	16

7.1.2 Material Colors	17

7.1.3 Textures	17

7.2 Importing from Lightwave	17

7.2.1 Importing Single Objects	17

7.2.2 Materials	18

7.2.3 Textures	18

7.2.4 Importing Scenes and Keyframe Data	18

7.2.5 Keyframing	18

7.2.6 Using PRO files instead of LWO files	19

8.0 Creating Entities	20

9.0 Cameras	21

10.0 Lights	23

11.0 Textures	24

12.0 Rendering a Scene	25

13.0 Animating Entities	26

13.1 Morphing Animation	26

13.2 Keyframe Animation	27

14.0 3D Sprites	29

14.1 Introduction	29

14.2 Making a sprite	29

14.3 Animating Sprites	29

15.0 Using the Terrain Engine	31

15.1 Introduction	31

15.2 Setting up the Tiles	31

15.3 Creating the Terrain Object	32

15.4 Loading the Terrain Data	32

15.5 Rendering the Terrain	32

15.6 Morphing Heighfields	33

16.0 Direct3D Notes	34

16.1 Initializing Direct3D	34

16.2 Rules to Remember about Direct3D	34

17.0 Power Render Utilities	35

17.1 PRO File Utilities	36

17.1.1 3DS2PRO	36

17.1.2 DUMPPRO	36

17.1.3 JOINPRO	37

17.1.4 UNIFY	37

17.1.5 OBJFLAG	37

17.1.6 SAVEMESH	38

17.1.7 LOADMESH	38

17.1.8 LWO2PRO	38

17.1.9 CENTPRO	38

17.1.10 LWS2PRO	39

17.1.11 PHONG	39

17.2 Texture Utilities	40

17.2.1 TEXSKIN	40

17.2.2 TEXSET	43

17.2.3 SPRMIP	44

17.3 Material Utilities	45

17.3.1 EDITMAT	45

17.3.2 MAKETAB	45

17.4 Terrain Utilities	46

17.4.1 PIC2WMP	46

17.5 Object Viewers	47

17.5.1 PRVIEW	47

# 1.0 Introduction

Welcome to Power Render, a 3D rendering API for PC's running DOS and Microsoft Windows 95/NT.

Since the beginning of computer graphics, there has been much effort put into 3 dimensional images.  The ability to move around and interact with a virtual world is a great experience compared to the dull 2D desktops that many people work with today.  Power Render is a development tool that will aid you in creating virtual worlds and 3D objects with stunning realism.

The trial version of Power Render version has been given out to the public for personal evaluation.  If you wish to continue using Power Render after one month of evaluation, you must license the software.  The licensed version of Power Render will remove the startup logo from your programs and utilities.

3Dfx Interactive has been nice enough to provide the Glide libraries for programmers, however you still need to know about 3D graphics techniques to use it.  Power Render gives you the rest of the routines needed for 3D graphics.  In addition to 3Dfx chip support, fast software rendering routines are provided which allow you to switch between hardware and software rendering with little changes to your code.

This release also supports Direct3D with 3D Hardware.  Software Rendering through Direct3D is not supported, but you can use Power Render’s DirectDraw libraries for 8 bit video modes.

All comments and questions should be directed to egerter@egerter.com.   New utilities will be released for testing on http://www.egerter.com as they become available.

# 1.1 System Requirements

- Pentium CPU

- SVGA video card with 640x480x256 support

- Two button mouse

- 8 megabytes of memory recommended

- Watcom C/C++ for 32 bit DOS applications

- Watcom C/C++ or Microsoft Visual C/C++ for Windows 95 applications

- DirectX 5 for Direct3D support

- A 3D card optional. A card using a 3Dfx chip for Glide libraries or any 3D card with Direct3D support, with at least 4 megs of memory capable of zbuffering and texture mapping is suitable.

- 15 megs of Hard Drive space

- A 3D modelling program (one of 3D Studio, 3D Studio Max, Lightwave)

# 1.2 Installing Power Render

In order to compile the examples and write new applications with Power Render, you need a compiler.  For DOS applications, Watcom C/C++ (version 10.6 and up) is supported.

For Win32 applications, Watcom C/C++ (version 10.6 and up) and Microsoft Visual C/C++ (tested with v5.0) are supported.

Make sure you have unzipped this archive with the -d option to recreate the original  directory structure.

After unzipping, you MUST include the pr\bin or pr\winbin (for Win32 versions) directory in the path statement of your autoexec.bat file.  This will allow you to run the utilities from any directory.

For detailed instruction about recompiling the utilities and examples, read the install.txt file in the main \pr directory.

# 1.3 Overview

## 1.31 General Comments

Power Render 2.5 adds support for Direct3D, which is part of the DirectX libraries from Microsoft.  This will allow you to create applications for 3D hardware which supports Direct3D.

## 1.32 Power Render Terminology

Several terms are used throughout the documentation and the library itself.  It is important that you understand each of the following terms and how they relate to a Power Render application.

| Name | Description |
| --- | --- |
| Vertex | A point in 3D space, defined with (X,Y,Z) coordinates |
| Face | A planar surface made by connecting vertices. All faces in Power Render are triangles. A face has a material associated with it, which tells the renderer how to draw it. |
| Object | Defines a 3D shape. It can be divided into smaller pieces called segments. An object must have at least one segment. If an object is rotated, translated, or scaled, the segments relating to the object will change as well. |
| Segment | A set of vertices and faces. Each segment can be rotated, translated, and scaled independently of other segments. For example, these represent the limbs of a person, while an object would be the collection of all limbs related to that person. |
| Entity | An instance of an object in the world. An entity contains the orientation of the object and all of its segments. The entity contains a pointer to an object, so if you change one object, all entities using the object will reflect the change. For example, if you make an object representing a tank, then you can create multiple instances of the tank with several entities. The actual mesh data is only stored once (inside the object). Entities are what you tell Power Render to draw. |
| Device | The display device used to show the images. The devices currently supported are: DEVICE_VGA - Standard 320x200x8bit VGA mode DEVICE_SVGA – VESA 1.2, or VESA 2.0 modes with a linear frame buffer, or DirectDraw under Windows95 DEVICE_3DFX - A 3D accelerated card using 3Dfx’s Voodoo graphics chipset. DEVICE_D3D – Direct3D (most 3D cards) |
| Viewport | A rectangular region on the display device where a rendered image is shown. |
| Camera | A viewing position from within the 3D world. A camera contains the field of view, and far and near clipping plane information. A camera can be rotated like an entity, or it can possess source and target locations for pointing towards another location. |
| Light | Provides illumination for shaded rendering methods. A light can have either finite or infinite distance. A DIRECTIONAL_LIGHT has a source location and infinite distance, while a POINT_LIGHT has a source location, strength, and falloff attributes. A point light illuminates in a spherical shape around the center, with the center being the brightest location. |
| Texture | Used to add detail to a face. Any bitmap can be used as a texture. |
| Material | Defines how a face will look when rendered. Material attributes include color, texture, which sides are visible (front, back, or both), mip mapping information, environment mapping information, and the rendering method (flat, gouraud, textured, etc). |
| Orientation | Defines how an entity or camera is positioned in the world. It has translation, rotation, and scaling information. |
| Vector | A line in 3D space. If the length of the line is 1, it is called a normal vector. Vectors are used to define directions, such as the direction between a light and an entity. |
| Matrix | A matrix is an array of floating point numbers used to define 3D transformations. All matrices in Power Render are 4x4, although they are defined as a single array of numbers with indices 0-15. |
| Block | A rectangular bitmap that is captured from the video buffer and stored in system memory. It uses the format of the video buffer. |
| Palette | An array of colors used in 8 bit modes. |
| Screen | Either a block used as a drawing surface or the area in video memory used to draw on. |

# 1.4 Coordinate Systems

Power Render uses a left handed coordinate system.  This means the positive z-axis points away from the viewer, as shown in the following illustration:

In a left-handed coordinate system, rotations occur clockwise around any axis that is pointed at the viewer.

## 1.41 Object Space (3D)

Each object is defined using object or local coordinates.  Usually the object is centered around (0,0,0).  In the case of segments which require a different pivot coordinate, the segment may be centered around some other coordinate in object space.

## 1.42 World Space (3D)

Power Render uses a fixed world coordinate system, where (0,0,0) is the center of the world.  All entities have a position in world space.

## 1.43 Camera Space (3D)

Before an entity is rendered, it must be transformed to camera space.  Camera space is relative to the current camera being used.  A positive Z value means the coordinate is in front of the camera.

## 1.44 Viewport Space (2D)

Viewport space is relative to the top left corner of the viewport’s window.  (0,0) is the top left corner of the viewport.

## 1.45 Device Space (2D)

While viewport space refers to coordinates inside a rendering window, device space refers to the entire display on the monitor.  (0,0) is the top left corner of the screen.  The maximum coordinates depend on the screen resolution of the display device.

# 2.0 Initializing Power Render

Power Render uses a display list to keep track of the triangles rendered during each frame.  The PR_Initialize routine is used to tell the maximum number of triangles that can be stored in the list.  When the PR_NewFrame routines is called, this list is emptied.  A program can render entities and

the list will be grow until finally the PR_RenderFrame function is called to draw the triangles.

Software rendering stores every triangle in the list, until the PR_RenderFrame is called.  At this time, the list is sorted by the z coordinates using a fast radix sort.

Hardware rendering uses this list to store alpha blended triangles.  If a triangle requires alpha blending, it is stored in the list and later they are sorted and drawn in back to front order.  Triangles that do not require alpha blending are drawn immediately and only use zbuffering for sorting.

# 3.0 Device Detection and Initialization

A device is a piece of video hardware that is used to display the final rendered images.  This can be a 2D graphics board, or a 3D accelerated board.  The program must call detection and initialization routines for the device it wants to use.  The devices are numbered, and can be identified with the following defines:

#define DEVICE_ANY 0

#define DEVICE_NONE 0

#define DEVICE_VGA 1

#define DEVICE_SVGA 2

#define DEVICE_3DFX 4

#define DEVICE_MYSTIQUE 5  (Not available in this release)

#define DEVICE_D3D 6

The detection routines are called PR_DetectVGA, PR_DetectSVGA, and so on.  If the device is not found, the routine returns DEVICE_NONE.  Notice this is the same value as DEVICE_ANY.  This allows you to build a series of detection routines that try to find the devices in the order you want. You will probably want to check for native 3D accelerated boards first, then try software rendering if they are not found.

Since Power Render can be linked with support for individual 3D hardware, you must use preprocessor defines to tell which devices are going to be available.  For example if you don't have the 3Dfx Glide libraries, you cannot call the PR_Detect3Dfx routine because it will reference Glide functions.  This will cause undefined symbols when you link the program.

Power Render has a number of preprocessor defines for controlling what detection routines will be called.  The defines used are:

| Preprocessor define | Used for: |
| --- | --- |
| __3DFX__ | This will use DosGlide functions |
| MSGLIDE or WTGLIDE | This will use WinGLide functions under Microsoft C or Watcom C |
| MSDD or WTDD | These stand for Microsoft C DirectDraw, and Watcom DirectDraw. If the device is DEVICE_D3D, the PR_DetectD3D function will be called. If the device is DEVICE_VGA or DEVICE_SVGA, DirectDraw software rendering will be used. You should also note that if you are using Direct3D, you should set the PR_Settings.Hardware variable to TRUE before you call the PR_Initialize routine. This will enumerate DirectDraw devices other than the primary DirectDraw device. |

The initialize routines are called PR_InitializeVGA, PR_InitializeSVGA, PR_InitializeD3D, and PR_Initialize3Dfx.  These routines will set up pointers to the functions used by the device.  Each device has a unique function to render a triangle for example.  When you call this function, it will call a different function depending on the device initialized.  This means only 1 device can be initialized at a given time.

If you do not care about multiple device support, you can simply ignore the preprocessor defines and call the appropriate detection and initialization routines for that device.  However, if you want to support more devices or need others to compile your code with different devices, you should follow the code from the examples.

# 4.0 Reserving Space for Rendering Data

Power Render maintains an array of textures, tables, and materials. These are dynamically allocated at runtime so you must tell how many elements will be required in each array.  You can do this by calling the PR_AllocMaterials, PR_AllocTextures, and PR_AllocShadeTables routines.

You should pass each function the maximum number of elements you'll expect to use.  If you try to use more elements than you allocated space for, the program will either crash or not behave as expected.  The size of these arrays are only limited by the amount of memory you have.

# 5.0 Setting the Video Mode

When a device detection routine is successful, a list of available video modes is created.  This list is stored in the PR_VideoModes array. Your program can list the video modes as part of a setup screen and allow the user to pick one.  The video modes are sorted from smallest to largest. Picking the smallest mode or using 640x480 as the default mode will ensure the best compatibility.  You should remember that some high resolution modes might be supported by the video board, but the monitor may not be able to display them.

The PR_SetMode routine requires width, height, and refresh rate parameters. The refresh rate is not used.  Modes under DirectX automatically use the correct refresh rate regardless of the value you pass to PR_SetMode.

The following code will get the width and height of the first video mode in the list, and then attempt to set the mode with these dimensions.

vwidth = PR_VideoModes[0].width;

vheight = PR_VideoModes[0].height;

PR_SetMode (vwidth, vheight, 60);

# 6.0 Viewports

A viewport is a rectangular region on the screen where the 3D rendering will be shown.  A PR_VIEWPORT structure is used to describe this region. The PR_OpenViewport function is used to initialize a viewport, and describes the four corners of the region used. For example, to initialize a viewport using the full screen, assuming the vwidth and vheight variables hold the screen dimensions:

PR_OpenViewport (&viewport, 0, 0, vwidth-1, vheight-1, VIEW_PLAIN);

The last parameter is a flag, which is now obsolete.  You should always use the VIEW_PLAIN.

Since multiple viewports can be initialized, the PR_SetViewport routine is used to tell Power Render which one is active.  A PR_VIEWPORT structure called active_viewport holds information about the last viewport passed to PR_SetViewport.  You may also need to call PRGFX_Clip so the 2D functions will be clipped to the active viewport.  For example:

PR_SetViewport (&viewport);

PRGFX_Clip (active_viewport.topx,

active_viewport.topy,

active_viewport.bottomx,

active_viewport.bottomy);

# 7.0 Loading and Building Objects

Power Render assumes that real applications will be loading a 3D mesh from a file rather than building it from scratch through a bunch of code. You can build simple objects by settings up the structures and filling them with data yourself, however for detailed models with hundreds of triangles, keyframes, and hierarchy, this is not practical.  Power Render includes importing utilities for 3D Studio (.3DS) and Lightwave (.LWO and .LWS) files. Other formats can be used if you know the specifications for them and you can fill in the structures yourself.  Example 10 contains a series of macros that help you build an object from scratch.  This will allow you to build objects without knowing the internal data structures used by Power Render.

Loading from 3D Studio or Lightwave files directly from your program is possible but not recommended.  Power Render uses a proprietary file format called PRO (Power Render Object).  All of the Power Render utilities use this format.  The PRO format uses the same data structures used by the Power Render library, which means it loads faster than other formats, and contains information specific to the renderer.  Using the PRO format will allow you to manipulate objects in many ways with the utilities included.  You will be able to apply textures, control mipmapping and environment mapping, edit material properties and more, without the need to recompile any code.

The objects are stored in a PR_OBJECT structure.  This contains the actual list of vertices and faces used by the object.  The PR_LoadPRO function will load a PRO file into a PR_OBJECT structure.  Any materials, textures, and shadetables used by this object are automatically loaded.  If one of these elements already exists, it will not be loaded twice.  For example, if a single texture image is used by several PRO files, only 1 copy will be loaded into memory.  The same applies to shadetables and materials.  You must make sure that materials have unique names so the system can correctly determine if it has already been loaded.  If you load two PRO files that use the same material name, but the properties (such as color) are different, the first one loaded will be used.

The following section is a guide on how to import from Lightwave and 3D Studio successfully.

## 7.1 Importing from 3D Studio

The 3DS2PRO utility will convert a .3DS file to the Power Render Object (PRO) format.  Each object in the 3DS file will be treated as a segment in the PRO format.  A single "master" parent object is not required.  Each segment can be separate from the others, or part of a hierarchy.  The keyframing information for the objects and cameras will be converted and stored in the PRO file.  Dummy objects are supported in versions 2.5 and later. Multiple instances of an object are not supported.

### 7.1.1 Keyframing

Object keyframes must contain only transformation, scaling, and rotation keys. Other advanced features such as morphing are not supported.

Camera keyframes are fully supported, including position, target, FOV, and roll values.  The 3DS file must contain ONLY ONE camera.  Since Power Render does not allow unique keys for the target, FOV and roll, you must make sure each parameter has the same key numbers.  For example, if you have position keys at frames 1, 10, 20, and so on, you must have target, FOV and roll keys at 1, 10, 20, etc.  If you have any keys that do not match the frame numbers of the others, the converter will fail.

### 7.1.2 Material Colors

Material colors are taken from the diffuse component in the material settings. If this color is not set, the object will be completely black if you do not apply a texture.  The diffuse color is sometimes left black if the object is meant to use texture in 3D Studio.  The alpha value of the materials is not imported.

### 7.1.3 Textures

Texture coordinates can be imported if the texture is not tiled or mirrored in any way.  By default, the converter uses non-wrapping texture to support textures of any size.  If you are using a 3D card, any sized texture can be wrapped and wrapping texture coordinate will work.  However, if you are using software rendering, you must use a 256x256 texture for textures which are wrapped or mirrored.  Since the converter creates non-wrapping textures in the material, you must use the EDITMAT utility and change the rendering method before attempting to view the PRO file.

3D Studio MAX requires a special 3DS export plugin to correctly export the texture coordinates.

## 7.2 Importing from Lightwave

### 7.2.1 Importing Single Objects

The LWO2PRO utility will convert a .LWO file to the Power Render Object (PRO) format.  The resulting PRO file will contain a single segment.

### 7.2.2 Materials

Material colors are taken from the diffuse component.  You must define new materials and apply them to the polygons instead of using the default material.

### 7.2.3 Textures

Textures are not supported.  You must use the TEXSKIN and TEXSET utilities to place textures.

### 7.2.4 Importing Scenes and Keyframe Data

The LWS2PRO utility will convert a .LWS file to the Power Render Object (PRO) format.  Each object in the LWS scene will be treated as a segment in the PRO format.  A single "master" parent object is not required.  Each segment can be separate from the others, or part of a hierarchy.  The keyframing information for the objects and cameras will be converted and stored in the PRO file.

### 7.2.5 Keyframing

Object keyframes must contain only transformation, scaling, and rotation keys. Other advanced features such as bones are not supported.

Camera keyframes are supported by position and target values. Cameras must aim at a specific object, and not rely on the camera's X/Y rotation values.  Camera rolling is controlled by the Z rotation. You can use dummy objects if you want the camera to point at a location between objects.  Cameras attached to objects are not supported.

### 7.2.6 Using PRO files instead of LWO files

Since textures are not supported by the LWO2PRO and LWS2PRO utilities, it is useful to load PRO file in place of LWO files when reading the scene file. This allows you to texture the PRO files with Power Render's utilities, and merge the individual pieces into a single file with the keyframing information.  This helpful when a single textured object is used more than once in the scene, or when several scenes make use of the same objects. To use PRO files instead of LWO file, add the -p switch when you run LWS2PRO. Remember to use the same scale factor when you use LWO2PRO and LWS2PRO. A scale of around 200 is recommend for Lightwave Objects, if they originally fill the default editing area.

# 8.0 Creating Entities

An object just contains the mesh and face information describing an object. To actually render this object you also need to tell where it exists in the world, along the with scale and rotation.  This information is stored in a PR_ENTITY structure.  An entity is an instance of an object within

the 3D world.  Multiple entities can use the same object description.

An instance is initialized using the PR_CreateEntity routine.  It returns a pointer to a new entity which you can store in a variable or array.

A single scene can be composed of several entities.  For example you can keep dummy objects used for camera movement in one entity, a static background mesh in another, and some more for animated characters.

# 9.0 Cameras

Cameras are used to describe what will be shown in the viewport.  A camera has a location and direction in the 3D world.  The direction can be specified by a specific target location, or by rotation values.  Internally, there is always a target location.  If rotation values are used, a 3D vector is transformed by the camera rotation matrix and this is used as the target.

Cameras can be stored in a linked list.  The first camera in the list can be retrieved by calling the PR_GetFirstCamera function.  New cameras can be added to the list using the PR_AddCamera function.  This allows you to use the PR_FindClosestCamera routine.  For example, if you have a

race track with several TV cameras sitting around it, this function will return the closest one and you can aim it at a vehicle going around the track.

If a camera is loaded from a PRO file, it will be added to this list. It must then be attached to the entity it was loaded from.  Since this type of camera may contain keyframing information, it must know which segment to aim at.  You might have multiple instances of the object, so the PR_AttachCameraEntity function will tell it which entity contains the segment it should use for a target.

You can also initialize a camera yourself, using the PR_InitializeCamera routine.  The camera mode determines if you are using rotation values or a specific target location.  This can be set using the PR_SetCameraMode routine with the CAMFLAG_AIM_TARGET or CAMFLAG_ANGLE_BASED values.

The PR_PositionCameraSource and PR_PositionCameraTarget routines are used to place the source and target locations.  The target location only applies if you are using the CAMFLAG_AIM_TARGET flag.

Multiple cameras can exist, so the PR_SetActiveCamera routine is used to tell Power Render which one will be used for the next frame.  You must call this routine whenever any attributes of the camera are changed, including position.

The camera controls how the rendered output will appear.  Properties include viewport size, aspect ratio, field of view, and clipping distances.  The aspect ratio is the ratio of the viewport width over the height.  Power Render does not calculate this itself, and uses 1.6 as a default.  This is the aspect ratio for a 320x200 or 640x400 window.  You can change this by accessing the aspect_ratio field of the camera structure.

The field of view is stored in radians.  The default value is Pi/3, or 60 degrees.  You can change this value by accessing the fov field of the camera structure.  Pi (180 degrees) is the maximum field of view possible.

The near and far clipping values control how far you can see into the horizon.  They must range between 1.0 and 65535.0.  These are the default values.  Lowering the far clipping distance can improve the speed since entities that lie completely behind this plane are not transformed or rendered.  You can change these values by accessing the nearclip and farclip fields of the camera structure.

# 10.0 Lights

Lights are stored in a structure call PR_LIGHTLIST.  You can use any number of light lists in your program.  To allocate a light list, use the PR_AllocLights command.  In addition to your own lights, you must also allocate a list and store it in the scenelights variable.  This contains the maximum number of lights present during the rendering of a frame.

When the PR_NewFrame function is called, the scenelights list is emptied.  New lights are added to the scene by the PR_TransformLights routine,  or if they are attached to an entity that is rendered.  It is recommended that you keep the lights in a list and maintain them yourself instead of attaching them to objects.  Objects with lights should be rendered first so subsequent entities will be lit by these lights.

There is no limit to the number of lights you can use in a scene, however you should set reasonable limits since too many will slow down the program.

# 11.0 Textures

Most textures used by Power Render are 8 bit images.  When using software rendering, all textures that are used in the same scene must share a common palette.  You should design your textures with this in mind.  You can also load 3DF texture which are specific to 3Dfx cards.

When using 3D hardware, textures can be stored in different formats.  The textures are still loaded from 8 bit images, however they can be converted to other bit depths as they are loaded into memory.  The PR_SetTextureFormat routine is used to set what format the textures will be converted to.  Each texture can use a different format simply by calling this function before it is loaded.  You should note that Direct3D chooses the closest format supported by the 3D device, so the format you set may not be the actual format used.

The formats available are:

TEXTURE_RGB_565		(16 bit RGB)

TEXTURE_ARGB_1555	(16 bit RGB with 1 alpha bit)

TEXTURE_P_8			(Palettized 8 bit)

TEXTURE_RGB_332		(8 bit RGB)

TEXTURE_ARGB_8332	(8 bit RGB with another 8 bits of alpha)

TEXTURE_ARGB_4444	(16 bit alpha+RGB)

TEXTURE_AP_88		(Palettized 8 bit with another 8 bits of alpha)

There are also two standard types called TEXTURE_NORMAL which is the same as TEXTURE_RGB_565, and TEXTURE_XRAY which is the same as TEXTURE_ARGB_1555.

If a format that uses alpha bit(s) is selected, the completely black color is treated as transparent.  The edges around regions of black space are outlined so that bilinear filtered sprites do not have black outlines around them.  Without this outline, bilinear filtering would cause the edges to blend towards the black color.

Palettized formats may be better than using the 16 bit RGB format, since the amount of texture memory is reduced.  However, under Direct3D some cards may slow down because it has to download a new palette each time a new texture is used.

Using paletted textures under Glide can be slower than using non-paletted texture since it has to download a new palette to the card for each image.  Power Render automatically determines if two palettes are the same when loading a new texture.  This reduces the number of times it has to download a new palette, since it knows if the selected texture used the same palette as the previous one.

Texture managent is handled internally by Power Render.  Any number of textures can be loaded even if they do not all fit in the 3D card's texture memory.  When a texture is required for rendering but is not in the texture cache, a least recently used algorithm determines which texture should be swapped out.

# 12.0 Rendering a Scene

Power Render does not include an entity list.  You must explicitly tell what entities to render and when.  This allows you to build your own routine that controls the entities.  With 3D hardware this can lead to faster frame rates, since you can mix artificial intelligence or other code between rendering calls.  While the card is rendering triangles, your code can be calculating the next frame.

At the beginning of a new frame, you must call the PR_NewFrame function.  Each entity must be transformed with the PR_TransformEntity routine, and rendered with the PR_RenderEntity routine.  The transformed vertex data exists in the PR_OBJECT structure, so you must not transform two entities that share an object description before rendering the first one.  After all entities have been rendered, call the PR_RenderFrame function.

Under Direct3D, no 2D graphics functions can be called between a PR_NewFrame and PR_RenderFrame pair.  Two exceptions to this are the PRGFX_ResizeTexture and PRGFX_PutTexture routines, which use triangles to draw a bitmap.  Direct3D also requires that only one PR_NewFrame and PR_RenderFrame pair is used for a frame, in order to correctly sort the polygons on some 3D cards.

# 13.0 Animating Entities

There are two ways of animating a model with Power Render.  The first method is vertex morphing, or vertex tweening.  The second method is keyframe animation.

## 13.1 Morphing Animation

Morphing works by modifying the vertex positions of a model.  Usually a model has a single list of vertices which define the shape.  A morphing object has two or more vertex lists which define the model in different positions or poses.  Each vertex list has the same number of vertices.  Vertices and faces cannot be added or removed between different frames of the animation.  All texture and face information remains the same for each frame in the sequence.  An example of morphing animation is used by Quake models.  Each frame shows a creature in a different pose, and animation is done by show a sequence of poses.  Power Render uses a more powerful animation technique called tweening.  Tweening can produce in-between frames of animation.  This allows for smoother animation between a series of vertex lists.

Power Render has two functions for morphing objects.  They are defined as follows:

void PR_Morph (PR_VERTEX *vert1, PR_VERTEX *vert2,

PR_VERTEX *vertdest, PR_DWORD numvert, PR_REAL t)

void PR_MorphWithNormals (PR_VERTEX *vert1, PR_VERTEX *vert2,

PR_VERTEX *vertdest, PR_DWORD numvert, PR_REAL t)

Vert1 and vert2 are pointers to two vertex lists.  Both functions will generate a set of vertices that is somewhere between the two vertex lists, and store it in vertdest, which is a previously allocated vertex list.  Usually vertdest will be a vertex list referenced by an existing object so the next time you render it, the vertices will have moved.  The t parameter is a floating point number between 0 and 1, which tells how close the animation is to the second vertex list.

Morphing with normals is required if the object uses shaded rendering methods and you want the lighting to look correct for the tweened vertex list.

Since morphing data is not part of the PRO format, you must store the frames (vertex lists) in a series of files.  Each frame of animation should be created with your modeling program and saved as a suitable import format.  After converting the frame to a PRO file, you can use the SAVEMESH utility to write the vertex list to a file.  The first frame should contain the texture mapping and material information.  This frame is loaded with PR_LoadPro and the vertex lists for other frames can be loaded using the code from the LOADMESH utility.  Once all of the data has been loaded, you can use the morphing routines mentioned above to generate the animation.  You should also make sure you save the first frame’s vertex list in a separate file because it will be modified when you call one of the routines.

This method requires a lot of data for each frame, however it allows you to deform the object in any way you like.

## 13.2 Keyframe Animation

The second method of animation uses a series of keyframes to tell the orientation of each segment.  Segments are given a specific translation, scaling, and rotation values at a certain frames of the animation.  The keyframe animation routines will calculate all intermediate frames for you.

Currently Power Render supports linear rotation between keys.  The maximum rotation between any two keys is 180 degrees.  Both linear and non-linear translation and scaling is supported.  Non-linear movement is achieved using splines, with bias, tension, and continuity information.  3D Studio also uses “ease to” and “ease from” information but Power Render ignores it.

With this type of animation segments are usually part of a tree structure.  This tree controls the relationships between different segments.  For example, if a hand is connected to an arm and the arm rotates, the hand will rotate as well.  This is called the parent-child relationship.  Each segment in the tree can have any number child segments, but only one parent segment.  Any transformation applied to the parent object will affect all of its children.

Each object can have any number of trees.  For each of these trees, there is one segment at the top.  This is called the root segment.  Any segment that is not a root segment is called a subsegment, and has the FLAG_SUBSEG bit set in the flags variable.  This is important because it tells the renderer that the segment will be drawn through recursive traversal of the tree.  If this flag is missing, each segment that references it as a child will draw it again.  Each segment has three pointers to other segments.  The child pointer is a link to the next segment in the tree.  If a segment has more than one child, it is linked by the next pointer. A pointer to the parent segment is also kept.

There are a number of things that can go wrong when importing keyframes from 3D Studio or Lightwave files.  Read the section on importing in chapter 7.  You should design your objects with these limitations in mind.  Here is a chart of the current limitations of the import formats:

| Feature: | 3D Studio (.3DS) | Lightwave (.LWS and .LWO) |
| --- | --- | --- |
| Spline Rotation Keys | No (only linear rotation) | No (only linear rotation) |
| Spline Translation Keys | Yes | Yes |
| Spline Scaling Keys | Yes | Yes |
| Dummy objects | Yes | Yes |
| Detail Polygons | Does not apply | No |
| N-sided polygons | Does not apply | No |
| Camera keyframes | Yes | Yes |
| Light keyframes | No | No |

The PR_AnimateEntity routine should be called before you transform each entity.  This will initialize the matrices in the segments for a particular frame of the animation.

For cameras that have keyframed movement, you can call the PR_AnimateCamera routine.  An example of this is shown in the PRVIEW source code.

# 14.0 3D Sprites

## 14.1 Introduction

3D sprites are essentially bitmaps that have a position and size in 3D space.  They always face directly at the current camera.  3D sprites let you use very complex objects without requiring a lot of polygons.  Some example uses of 3D sprites are background objects such as trees, or animated explosions.

## 14.2 Making a sprite

The first step is to create a material that will be used to display the sprite. For example:

PR_MATERIAL m;

PR_DWORD smoke_material;

PR_SetMaterialName (&m, "smoke sprite");

PR_SetMaterialMethod (&m, T_XTEXTURED);

PR_SetMaterialBaseColor (&m, 0);

PR_SetMaterialColor (&m, 255, 255, 255);

PR_SetMaterialTexture (&m, PR_LoadTexture("smoke.pcx"));

PR_SetMaterialShades (&m, 31);

PR_SetMaterialEnvironmentMap (&m, 0);

PR_SetMaterialMipMapState (&m, FALSE);

smoke_material = PR_AddMaterial (&m);

The next step is to create a sprite object.  Instead of loading an object from a PRO or a 3DS file, use a routine called PR_AllocSprite.  This creates an object with 2 triangles and 4 vertices.  The faces are set up so they form a 2D bitmap, with the same dimensions as the texture.  From this point on, the sprite is treated like a regular object.  You can create multiply instances of it by creating entities, and it works the same way as a 3D object.

You can scale the sprite using PR_ScaleEntity.  To rotate a 3D sprite, you must use the PR_RotateSegment routine with the first segment within the entity, and rotate about the Z axis.

Since a 3D sprite is composed of polygons, it can use shaded rendering methods such as Gouraud Texture.  Also, it makes no sense to use perspective texture mapping on a 3D sprite since the Z values at each vertex are the same.

## 14.3 Animating Sprites

When animating a 3D sprite, you should create an array of materials, and an array of objects created with PR_AllocSprite.  The reason for this is that you may need to have multiple instances of the sprite within the world.  Since the texture number is stored in the material, changing it will affect all sprites using that material.  Instead of changing the texture number, you can change the pointer to the object used by the entity. This pointer is stored in the “shape” variable of the entity.

For example:

explosion_entity[i]->shape = explosion_shape[frame_number];

# 15.0 Using the Terrain Engine

## 15.1 Introduction

The goal of the terrain engine is to provide a large world based on a 2D bitmap.  The landscape is large grid, and each value on the bitmap represents the height of a vertex on the grid.

Power Render uses up to 3 bitmaps for the landscape.  The first is a 8-bit bitmap which represents the height values (0-255).  The second is an 16-bit bitmap which represents the texture number (0-65535) for the square which is directly below and to the right of the vertex.  This bitmap is represented as a WGT map file so you can use the WGT Map Editor to design the surface features of the landscape.  The third bitmap is optional.  It is an 8-bit bitmap which represents the shading values for each vertex.  This allows you to precompute the lighting of the landscape using radiosity, raytracing, or other lighting techniques.  All of the bitmaps must be the same size.

Power Render does not include utilities for creating the heightfield or other bitmaps.  You can use a third party terrain generator, or create a utility which can edit the landscape by modifying heights, textures, and shade values.

## 15.2 Setting up the Tiles

You must load an array of textures for the tiles.  You can store each texture in a separate file, or you can keep them in a WGT sprite file.  I prefer the latter since all the images are kept in a single file and can be loaded with the wloadsprites routine.  If you use this method, each texture must be added to the world texture list.  See the prland.c example for a complete example.

The first material should use the NULL rendering type.  This will allow certain parts of the landscape to be invisible when using multiple levels.

m = &PR_ObjectMaterialList[OBJ_TILES];

PR_SetMaterialName (m, "NULL");

PR_SetMaterialMethod (m, NULL_TYPE);

OBJ_TILES is a number which tells how much room to leave for other materials.  When a new material is added, it picks the next available index.  We don’t want this to use materials that the terrain might be using, so some indices are left empty.  You tell the terrain engine what the first index used by the tiles is when you call the PR_AllocateTerrain routine.

Each texture must have a unique material associated with it.  You can set up the material’s properties differently for each texture.  If you are using shading, the rendering method should include flat or gouraud shading.  Mipmapping may increase the speed of the terrain.  You can also switch between gouraud and flat shading based on the distance.

## 15.3 Creating the Terrain Object

A special object is used for terrains.  You can generate the object be using the PR_AllocateTerrain function.  For example:

terrain = PR_AllocateTerrain (terrain_size,

TERRAIN_PLAIN | TERRAIN_SHADING,

WORLD_SCALE, HEIGHT_SCALE, OBJ_TILES);

The terrain object is a square grid, of (N+1)x(N+1) vertices and NxN*2 faces.  The size of the terrain is passed in the first parameter.  This is actually half the length of the terrain mesh (N).  This number determines how many pieces of the grid mesh be be seen away from the camera. The camera always remains in the center of this grid mesh.

## 15.4 Loading the Terrain Data

To load the 3 bitmaps required to display a heightfield, call the PR_LoadTerrain function.  The shading bitmap is optional.  For example:

PR_LoadTerrain (terrain, 1, "height.pcx", "map.wmp", "shade.pcx", 32);

The last parameter tells the maximum shade value used in the shading bitmap.  Up to 3 sets of bitmaps can be loaded at once, to form multiple layered heightfields.  The second layer is upside down, for cave ceilings.  With 2 layers you can create completely underground terrain maps.  The third layer is right side up like the first layer, and is used for the top exterior of a cave.  With three layers you can create maps with both outdoor and cave regions.

When using multiple layers, you will not want to see all layers everywhere.  For example you might have a lake area which only requires the first layer.  If you make the first tile number use the NULL rendering method, the polygons will be immediately rejected and will not draw.  The heightfields for the other layers still have values, but they will not be shown.

## 15.5 Rendering the Terrain

The terrain heightfield data is read each time the camera moves one grid length.  For example, if the camera is simply spinning and not translating, the heightfield, shading, and tile data does not change.  You must call the PR_UpdateTerrain each frame, between the time you move the camera and you transform the terrain.

When you want to render the terrain, call the PR_TransformTerrain.  This will transform and render the terrain at the same time.  All layers currently loaded will be transformed.  The terrain library uses special optimizations which allows for multiple layered terrains to be transformed quickly.

## 15.6 Morphing Heighfields

To modify values on the terrain, you can access the heightfield as a bitmap and draw on it.  Since the mesh data is only changed periodically, you must call the PR_ModifyTerrain function to let the library know it should read new heightfield data when PR_UpdateTerrain is called.

The terrain library calls a function PR_TerrainWaveFunction when each set of values is read from the terrain bitmaps.  This allows you to change the height, shading, and tile data depending on the x and y coordinates of the vertex. This is useful for making rippling wave patterns or pulsating lights in sections of the landscape.  The wave function can do different things depending on the tile number, so any number of effects can be used on the same terrain.  This function must be present in your application regardless if you use it.  If you do not want waves, you can simply make the function return immediately.  Using this function requires that you constantly call the PR_ModifyTerrain function so the new terrain data will be read into the terrain mesh.

The terrain appears to be an endless world.  You can fly from one side of it to another and it repeats itself.  This means the height values along the edges of the heightfield should be similar to those along the opposite side. You must make sure the camera wraps around at the edges of the world, and also that any entity is no more than half the map width and height away from the camera.  There is code in the terrain examples that show how to fix the coordinates of entities when the camera location wraps around.

# 16.0 Direct3D Notes

Power Render version 2.5 and later supports most 3D accelerator cards by Direct3D.  DirectX 5.0 or later is requried, and the display drivers must support the DrawPrimitive routine.

## 16.1 Initializing Direct3D

The examples are set up so Direct3D is used by default.  You can specify Direct3D or Software rendering through DirectDraw by changing the device variable.  DEVICE_ANY will find the first available device, with 3D accelerated devices preferred.  If no suitable Direct3D enabled cards are available, it will use Power Render’s software rendering routines.  Software Rendering through Direct3D is currently not supported.

You can force Direct3D or software rendering by setting the device variable to DEVICE_D3D or DEVICE_SVGA respectively.

## 16.2 Rules to Remember about Direct3D

Some 3D cards do no support 2D routines while rendering 3D triangles.  This means all routines which Blt or Lock a surface cannot be used between a PR_NewFrame and PR_RenderFrame pair.  All of the PRGFX_* routines except for PRGFX_PutTexture and PRGFX_ResizeTexture fall into this category.  The routines for displaying textures draw a rectangle formed by two triangle, and must be between a PR_NewFrame and PR_RenderFrame pair.

The PRGFX_Clearscreen routine will only clear the backbuffer.  If you need to clear the front buffer, use the PRGFX_Bar routine instead.  This should only be used for non-realtime code, since it accesses the linear frame buffer directly.

The terrain engine requires that heightfields and their associated bitmaps have a width that is a multiple of 8.

# 17.0 Power Render Utilities

There are many utilities included which make it easier to manipulate the data required for showing 3D worlds.  These programs can be divided into 5 categories:  PRO File Utilities, Texture Utilities, Material Utilities, Terrain Utilities, and Object Viewers.

## 17.1 PRO File Utilities

### 17.1.1 3DS2PRO

Purpose:	Convert 3D Studio .3DS files into the PRO format.

Usage:		3DS2PRO filename.3DS [-s #] [-t EXT]

Options:	-s #		: Set the conversion scale (1.0 for no scaling)

-t EXT		: Sets the texture extension, for textures obtained from

the 3DS file.

Before using the other Power Render utilities, you must first convert your models into .PRO files.  If you do not use the -t option, you must define the textures yourself using TEXSKIN or TEXSET.  All texture files must be one of the formats supported by Power Render.  When converting, the textures must be in the same directory as the 3DS file.  If the texture wraps around any faces more than once, make sure you use a 256x256 texture and change the texture mapping method accordingly.

For some of the models that came with the 3Dfx Flip example, you can use -t 3DF to use the 3DF textures provided with them.

The scale parameter is used to alter the original scale of the object.  If you want it to contain the same coordinates as the modeling program, use a scale of 1.  Scaling the mesh data is preferred to scaling the matrices within your program.

### 17.1.2 DUMPPRO

Purpose:	Provides a text dump for viewing the contents of a PRO file.

Usage:		DUMPPRO filename.PRO

This is mainly provided for debugging the file format.  Several pages of information will be shown so you should redirect the output.

For example, DUMPPRO robot.pro > robot.txt

### 17.1.3 JOINPRO

Purpose:	Joins two PRO files into one.

Usage:		JOINPRO file1.PRO file2.PRO outfile.PRO

This utility allows you to create multiple textured objects by joining two unique objects into one.  The resulting file will be treated as a single object with new segments.

### 17.1.4 UNIFY

Purpose:	Combines all segments into one

Usage:		UNIFY file1.PRO outfile.PRO

This utility will merge the vertex and face lists of all the segments and make one segment with everything in it.  It is typically used after JOINPRO when constructing an object.

### 17.1.5 OBJFLAG

Purpose:	Changes the visibility flags of an object.

Usage:		OBJFLAG file1.PRO outfile.PRO

This utility allows you to change the how the faces of an object will be shown.  You can choose one of front visible, back visible, or both.  Every polygon in the object will be modified with the new flags.

### 17.1.6 SAVEMESH

Purpose:	Saves the vertices of an object to a raw data file

Usage:		SAVEMESH filename.pro mesh.PRM

Sometimes you may wish to bend or flatten a mesh in order to map it correctly using TEXSKIN.  SAVEMESH and LOADMESH allow you to import and export the coordinates of the vertices without changing the texture data.

### 17.1.7 LOADMESH

Purpose:	Loads the vertices from a raw data file into an object

Usage:		LOADMESH filename.pro mesh.PRM

This utility will replace the vertices in the PRO file with those in the PRM file.  Only the coordinates of the vertices are imported.

### 17.1.8 LWO2PRO

Purpose:	Converts a Lightwave object file to a PRO object file.

Usage:	LWO2PRO is operates the same way as 3DS2PRO, except that textures  are not supported.  Only triangle based objects are supported.

### 17.1.9 CENTPRO

Purpose: 	Centers all vertices within an object around the origin.

Usage:		CENTPRO infile.PRO outfile.PRO

This utility should not be used with object containing a hierarchy and pivot locations.

### 17.1.10 LWS2PRO

Purpose:	Converts a Lightwave scene file to the PRO format

Usage:		LWS2PRO file.LWS file.PRO [-p] [-s #]

This utility converts a Lightwave Scene file into a PRO file.  Keyframe animation is also converted.  The –p option will load .PRO files instead of .LWO files when reading the scene.  This allows you to apply different textures and materials to individual parts of the scene.

### 17.1.11 PHONG

Purpose:	Doubles the number of faces in an object and makes the second set environment mapped with a phong highlight texture map.

Usage:		PHONG  infile.PRO outfile.PRO

This utility is meant for use with 3D cards only.  All faces in the object will be duplicated and set up for alpha blending.  A 256x256 PCX image called phong.pcx must be in the directory of the object.

## 17.2 Texture Utilities

### 17.2.1 TEXSKIN

Purpose:	Apply textures to a PRO file, using the planar mapping method.

Usage:		TEXSKIN filename.PRO front.[IMG] [back.[IMG]] [-m materialname]

One of the hardest parts of 3D game design is getting the textures place on the characters and objects.  Most 3D modeling applications allow for planar, cylindrical, and spherical mapping.  This requires a lot of patience because you cannot see the results of the mapping immediately.  Instead you have to wait for the application to render the scene.  Power Render allows you to interactively align textures onto the objects.  You can easily set the scale, rotation plane, and offset of the texture.

As well as mapping a single texture onto a 3D object, you can also use front and back textures.  An adjustable back plane determines which texture a polygon should use.  This is a powerful option for creating characters.  For example you would not want a person's face to be on the back of his/her head, so you can apply a front and back texture.  This method works best if the mesh is laid out flat and the back plane intersects the mesh at the correct places.  The back texture is optional.  If you do not specify a back texture on the command line, a single texture is used for all polygons.  Front and back textures MUST be the same dimensions.

The –m switch allows you to set the texture for a particular material.  Existing textures and materials are not changed.  The material will be reset to a plain texture method, so you must later run EDITMAT to change the method.  You can also use the EDITMAT utility to find out the material names.

When you run this utility, the object will appear in the top left of the screen, and a control panel will be shown on the right side.

There are several variables that you can control using + and - buttons.  They are:

X:		- X plane rotation

Y:		- Y plane rotation

Z:		- Z plane rotation

X Scale:	- X scale of texture

Scale:		- Changes both X and Y scale of texture

Y Scale:	- Y scale of texture

Back Dist:	- Z value of the back plane (only appears if using two textures)

Note that clicking on the right button of the mouse will change the variables much quicker, and the object is not updated until you release the button.  Below these variables are two buttons. The first is for aligning the texture to the viewport.  This make it easy to align a texture.  First rotate the object so the front is facing you, then click on this button or hit the 'A' key.  The texture plane will be directly facing you now.  Note that you can hold down the 'A' key as you rotate the object and texture will remain facing you.

The next button is used to control texture wrapping.  If your texture has a width of 256 pixels, you can make the texture repeat many times over the surface of the 3D object.  When using 3D hardware, any sized texture can be wrapped, however the object will not work with software rendering.

The next set of buttons are the control mode buttons.  When you click and drag the mouse inside the object preview window, you change one of the viewing variables.  Only one of these buttons can be selected at once.  They are: Rotate Object, Move Object, Rotate Plane, Scale, and Camera.  Each button has a keyboard shortcut so you don't have to click on the buttons to change modes.

Rotate Object is self explanatory.  The texture plane will rotate along with the object so no changes will be made to the mapping.

Controls:

Left click = X/Y rotation, Right click = Z rotation

Keyboard shortcut is 'Spacebar'

Move Object will slide the object along the X or Y axis.  The texture plane does not move.  This allows you to offset the texture from the middle of the mesh.

Controls:

Left click = X/Y translation, Right click = nothing

Keyboard shortcut is 'Left Shift'

Rotate Plane operates the same way as Rotate Object, only the object remains still and the plane rotates.

Controls:

Left click = X/Y rotation, Right click = Z rotation

Keyboard shortcut is 'Left Ctrl'

Scale changes both X and Y scale values.

Controls:

Left click = Change X/Y scale, Right click = nothing

Keyboard shortcut is 'S'

You can also hold down ‘X’ or ‘Y’ for scaling in a single axis only.

Camera allows you to move around the object without affecting the mapping coordinates.

Controls:

Left click = Translate X/Y, Right click = Translate Z (zoom)

Keyboard shortcut is 'Z'

Once you're happy with the results, you can click on save.  This will overwrite any texture coordinates previously stored in the PRO file.

### 17.2.2 TEXSET

Purpose:	Allows you to change the texture position and shade of each vertex in every polygon.

Usage:		TEXSET filename.pro

This utility allows you to place the texture coordinates exactly where you want them.  You will typically use this after applying a texture with TEXSKIN, or to define texture positions from scratch.  You can also set the precalculated lighting values.

The interface is similar to the TEXSKIN utility except for the toolbar on the right side.  The first five variables you can change deal with the segment, face, vertex, material, and shade.  Below these you will see a texture preview window.  A triangle will display in this window to show exactly how the texture coordinates are mapped on the triangle in the mesh.  A corresponding triangle will appear in the Object Preview window to highlight the face you are modifying.

Note that tiled textures are currently not supported.

The shade value is a range between 0 and 100 percent.  The actual value stored in the file depends on the number of shades used by the material.

Below the texture window are two toggle switches for the Object Preview window.  These allow you to rotate the object, and move the camera around.  The rotate button’s keyboard shortcut is the spacebar, and the camera button’s shortcut is the ‘Z’ key.

### 17.2.3 SPRMIP

Purpose:	Creates mip maps for a series of textures

Usage:		SPRMIP filename.spr

This utility will make mip map textures given a WGT Sprite file containing the original images.  Four levels are created and stored back into the file.  This makes it easy to modify a texture, because you can update the mip maps as well.

Note that in order to import a large texture, you must use the LOAD SINGLE option within the Sprite Editor.  This will load any image into a slot and ignore the palette.

The utility will ask how many textures are stored in the file.  You should enter the maximum number of normal textures you expect to use.  For example if you choose 256, the mip maps will be created in the following sprite positions:

0-255:		Original textures (left intact)

256-511:	Mip map level 1 (1/2 size)

512-767:	Mip map level 2 (1/4 size)

768-1023:	Mip map level 3 (1/8 size)

1024-1278	Mip map level 4 (1/16 size)

Mip maps are stored as separate images because the smaller textures will fit in the cache better and therefore draw quicker.

## 17.3 Material Utilities

### 17.3.1 EDITMAT

Purpose:	Edits the materials used by an object

Usage:		EDITMAT filename.PRO

This utility allows you to change the material properties of an object.  You can add new materials, textures, or shade tables, as well as modify the existing ones.

There are 3 modes which are chosen by the buttons at the top right of the screen.  The modes are “material”, “texture”, and “shade table”.

The attributes you can modify are:

Name

Render Method

Texture Number

Base Color

Shades

Shade Table Number

Environment Map State

Environment Map Axis

Mip Map State

Mip Map Material

Mip Map Shift

RGBA color information

Texture Filename

Shade table Filename

### 17.3.2 MAKETAB

Purpose:	Creates shade tables using a palette contained in a texture map.

Usage:		MAKETAB texture table

The utility will first ask you for the number of shades.  More shades will provide greater shading detail and realism.  Values of 32 or 64 are commonly used.

You will then be asked for the type of table you want to create.  Valid types are Gouraud, Phong (Gouraud with a specular component), and Translucent.

If you choose Gouraud, you will be asked for the starting and ending percentage of the colors.  A normal Gouraud range would be from 0.0 to 1.0.  You can add ambient lighting by starting at 0.2 instead of 0.0 for example.  You can also add over brightening, such as 0.0 to 3.0. This means the brightest color in the table will be 3 times as bright as the original color.

If you choose Phong, you will be asked for the Specular and Shine values.  Typical Specular amounts range between 0.0 and 1.0.  Typical Shine amounts range between 20 and 1000.  The higher the shine value, the more concentrated the highlight is.  If you choose a shine value below 20, you'll end up with a very bright object.  For starters, try the values 0.5 and 100.

If you choose Translucent, you will be asked for percentages of foreground and background colors.  Both values should total to 1, but they can be anything.  For example you could use 2.0 (200%) of the foreground, and 0.3 (30%) of the background.  The MAKETAB utility doesn't care what values you use so it's up to you to make the translucency look nice.

The texture is shown on the screen as the table is being calculated, and the table is shown visually once it is computed.  A keypress will then return you to DOS.  Please be patient because the table requires a lot of time to calculate.

## 17.4 Terrain Utilities

### 17.4.1 PIC2WMP

Purpose:	Convert a PCX image to a WGT map.

Usage:		PIC2WMP filename.pcx outfile.wmp

This utility can be used to turn any PCX image into a WMP file which is used to store the tile numbers for the terrain.  A WMP file can be edited by the WGT map editor.  The main advantage of using WMP files is that you can have more than 256 unique tiles, unlike an 8 bit PCX image.

## 17.5 Object Viewers

### 17.5.1 PRVIEW

Purpose:	Display 3DS and PRO files interactively.

Usage:		PRVIEW filename.[PRO/3DS/LWO/MDL] [options]

Options available are:

-s scale		Changes scaling factor

-t ext		Imports textures from 3DS, using extension

ext = [PCX, 3DF, IFF, BLK, PAK]

-m pakfile	Specifies a PAK file (ie pak1.pak)

If none specified, it loads from the BSP file directly

-v		Uses standard 320x200 VGA mode instead of VESA

3DS files are shown with Gouraud shading, and the palette is calculated from the diffuse components of the materials.

Controls:

A			- move forward

Z			- move backward

minus/plus		- change viewport size

F1			- Cycles through the available video modes

ESC			- Quits

Move mouse		- Tilt camera

A			- Move camera forward

Z			- Move camera backward

+ (keypad)		- Increase movement speed

- (keypad)		- Decrease movement speed

Left Mouse Button + move mouse

- Spin the model

Quickly let go while moving

- Keeps the model spinning

(This is quite sensitive and hard to do if the model is simple)

Spacebar		- Toggle MDL animation on/off

F1			- Cycles through VESA video modes

ESC			- Quits

[			- Slows down animation

]			- Speed up animation

;			- Decrease gamma correction (3Dfx only)

'			- Increase gamma correction (3Dfx only)

<			- Decrease ambient lighting

>			- Increase ambient lighting