******************************************************************************

Here is the standard midifile format description. This is also available

from ucsd.edu as midi/doc/midi-files.Z

Patrick Aalto

ap@tukki.jyu.fi

-------------- CUT HERE --------------

[This document is Dave Oppenheim's current version of the MIDI file

specification, as sent to those who have participated in its

development.  The consensus seems to be to submit this to the MIDI

Manufacturers' Association as version 1.0.  I apologize for any loss of

clarity that might have occurred in the conversion from a Microsoft Word

document to this pure text file.  I have removed some of the discussion

about recent changes to the specification in order to keep the file size

reasonable.--Doug Wyatt]

Standard MIDI Files 0.06        March 1, 1988

0  Introduction

This describes a proposed standard MIDI file format.  MIDI files contain

one or more MIDI streams, with time information for each event.  Song,

sequence, and track structures, tempo and time signature information,

are all supported.  Track names and other descriptive information may be

stored with the MIDI data.  This format supports multiple tracks and

multiple sequences so that if the user of a program which supports

multiple tracks intends to move a file to another one, this format can

allow that to happen.

This spec defines the 8-bit binary data stream used in the file.  The

data can be stored in a binary file, nibbleized, 7-bit-ized for

efficient MIDI transmission, converted to Hex ASCII, or translated

symbolically to a printable text file.  This spec addresses what's in

the 8-bit stream.

1  Sequences, Tracks, Chunks:  File Block Structure

Sequence files are made up of chunks.  Each chunk has a 4-character type

and a 32-bit length, which is the number of bytes in the chunk.  On the

Macintosh, data is passed either in the data fork of a file, or on the

Clipboard.  (The file type on the Macintosh for a file in this format

will be "Midi".)  On any other computer, the data is simply the contents

of the file.  This structure allows future chunk types to be designed

which may easily be ignored if encountered by a program written before

the chunk type is introduced.   Your programs should expect alien chunks

and treat them as if they weren't there.

This proposal defines two types of chunks:  a header chunk and a track

chunk.  A header chunk provides a minimal amount of information

pertaining to the entire MIDI file.  A track chunk contains a sequential

stream of MIDI data which may contain information for up to 16 MIDI

channels.  The concepts of multiple tracks, multiple MIDI outputs,

patterns, sequences, and songs may all be implemented using several

track chunks.

A MIDI file always starts with a header chunk, and is followed by one or

more track chunks.

MThd  <length of header data>

<header data>

MTrk  <length of track data>

<track data>

MTrk  <length of track data>

<track data>

...

Track Data Format (MTrk chunk type)

The MTrk chunk type is where actual song data is stored.  It is simply a

stream of MIDI events (and non-MIDI events), preceded by delta-time

values.

Some numbers in MTrk chunks are represented in a form called a variable-

length quantity. These numbers are represented 7 bits per byte, most

significant bits first.  All bytes except the last have bit 7 set, and

the last byte has bit 7 clear.  If the number is between 0 and 127,  it

is thus represented exactly as one byte.

Here are some examples of numbers represented as variable-length

quantities:

Number (hex)    Representation (hex)

00000000        00

00000040        40

0000007F        7F

00000080        81 00

00002000        C0 00

00003FFF        FF 7F

00004000        81 80 00

00100000        C0 80 00

001FFFFF        FF FF 7F

00200000        81 80 80 00

08000000        C0 80 80 00

0FFFFFFF        FF FF FF 7F

The largest number which is allowed is 0FFFFFFF so that the variable-

length representation must fit in 32 bits in a routine to write

variable-length numbers.  Theoretically, larger numbers are possible,

but 2 x 108 96ths of a beat at a fast tempo of 500 beats per minute is

four days, long enough for any delta-time!

Here is the syntax of an MTrk chunk:

<track data> = <MTrk event>+

<MTrk event> = <delta-time> <event>

<delta-time> is stored as a variable-length quantity.  It represents the

amount of time before the following event.  If the first event in a

track occurs at the very beginning of a track, or if two events occur

simultaneously, a delta-time of zero is used.  Delta-times are always

present.  (Not storing delta-times of 0 requires at least two bytes for

any other value, and most delta-times aren't zero.)  Delta-time is in

some fraction of a beat (or a second, for recording a track with SMPTE

times), as specified in the header chunk.

<event> = <MIDI event> | <sysex event> | <meta-event>

<MIDI event> is any MIDI channel message.  Running status is used:

status bytes may be omitted after the first byte.  The first event in a

file must specify status.  Delta-time is not  considered an event

itself:  it is an integral part of the specification.  Notice that

running status occurs across delta-times.

<meta-event> specifies non-MIDI information useful to this format or to

sequencers, with this syntax:

FF <type> <length> <bytes>

All meta-events begin with FF, then have an event type byte (which is

always less than 128), and then have the length of the data stored as a

variable-length quantity, and then the data itself.  If there is no

data, the length is 0.  As with sysex events, running status is not

allowed.  As with chunks, future meta-events may be designed which may

not be known to existing programs, so programs must properly ignore

meta-events which they do not recognize, and indeed, should expect to

see them.  New for 0.06:  programs must never ignore the length of a

meta-event which they do recognize, and they shouldn't be surprised if

it's bigger than they expected.  If so, they must ignore everything past

what they know about.  However, they must not add anything of their own

to the end of a meta-event.

<sysex event> is used to specify a MIDI system exclusive message, or as

an "escape" to specify any arbitrary bytes to be transmitted.

Unfortunately, some synthesizer manufacturers specify that their system

exclusive messages are to be transmitted as little packets.  Each packet

is only part of an entire syntactical system exclusive message, but the

times they are transmitted at are important.  Examples of this are the

bytes sent in a CZ patch dump, or the FB-01's "system exclusive mode" in

which microtonal data can be transmitted.  To be able to handle

situations like these, two forms of  <sysex event> are provided:

F0 <length> <bytes to be transmitted after F0>

F7 <length> <all bytes to be transmitted>

In both cases, <length> is stored as a variable-length quantity.  It is

equal to the number of bytes following it, not including itself or the

message type (F0 or F7), but all the bytes which follow, including any

F7 at the end which is intended to be transmitted.  The first form, with

the F0 code, is used for syntactically complete system exclusive

messages, or the first packet an a series Q that is, messages in which

the F0 should be transmitted.  The second form is used for the remainder

of the packets within a syntactic sysex message, which do not begin with

F0.  Of course, the F7 is not considered part of the system exclusive

message.  Of course, just as in MIDI, running status is not allowed, in

this case because the length is stored as a variable-length quantity

which may or may not start with bit 7 set.

(New to 0.06)  A syntactic system exclusive message must always end with

an F7, even if the real-life device didn't send one, so that you know

when you've reached the end of an entire sysex message without looking

ahead to the next event in the MIDI file.  This principle is repeated

and illustrated in the paragraphs below.

The vast majority of system exclusive messages will just use the F0

format.  For instance, the transmitted message F0 43 12 00 07 F7 would

be stored in a MIDI file as F0 05 43 12 00 07 F7.  As mentioned above,

it is required to include the F7 at the end so that the reader of the

MIDI file knows that it has read the entire message.

For special situations when a single system exclusive message is split

up, with parts of it being transmitted at different times, such as in a

Casio CZ patch transfer, or the FB-01's "system exclusive mode", the F7

form of sysex event is used for each packet except the first.  None of

the packets would end with an F7 except the last one, which must end

with an F7.  There also must not be any transmittable MIDI events in-

between the packets of a multi-packet system exclusive message.  Here is

an example:  suppose the bytes F0 43 12 00 were to be sent, followed by

a 200-tick delay, followed by the bytes  43 12 00 43 12 00, followed by

a 100-tick delay, followed by the bytes  43 12 00 F7, this would be in

the MIDI File:

F0 03 43 12 00

81 48                                   200-tick delta-time

F7 06 43 12 00 43 12 00

64                                      100-tick delta-time

F7 04 43 12 00 F7

The F7 event may also be used as an "escape" to transmit any bytes

whatsoever, including real-time bytes, song pointer, or MIDI Time Code,

which are not permitted normally in this specification.  No effort

should be made to interpret the bytes used in this way.  Since a system

exclusive message is not being transmitted, it is not necessary or

appropriate to end the F7 event with an F7 in this case.

2    Header Chunk

The header chunk at the beginning of the file specifies some basic

information about the data in the file.  The data section contains three

16-bit words, stored high byte first (of course).  Here's the syntax of

the complete chunk:

<chunk type> <length> <format> <ntrks> <division>

As described above, <chunk type> is the four ASCII characters 'MThd';

<length> is a 32-bit representation of the number 6 (high byte first).

The first word, format, specifies the overall organization of the file.

Only three values of format are specified:

0       the file contains a single multi-channel track

1       the file contains one or more simultaneous tracks (or MIDI

outputs) of a sequence

2       the file contains one or more sequentially independent

single-track patterns

The next word, ntrks, is the number of track chunks in the file.  The

third word, division,  is the division of a quarter-note represented by

the delta-times in the file.  (If division is negative, it represents

the division of a second represented by the delta-times in the file, so

that the track can represent events occurring in actual time instead of

metrical time.  It is represented in the following way:  the upper byte

is one of the four values -24, -25, -29, or -30, corresponding to the

four standard SMPTE and MIDI time code formats, and represents the

number of frames per second.  The second byte (stored positive) is the

resolution within a frame:  typical values may be 4 (MIDI time code

resolution), 8, 10, 80 (bit resolution), or 100.  This system allows

exact specification of time-code-based tracks, but also allows

millisecond-based tracks by specifying 25 frames/sec and a resolution of

40 units per frame.)

Format 0, that is, one multi-channel track, is the most interchangeable

representation of data.  One application of MIDI files is a simple

single-track player in a program which needs to make synthesizers make

sounds, but which is primarily concerned with something else such as

mixers or sound effect boxes.  It is very desirable to be able to

produce such a format, even if your program is track-based, in order to

work with these simple programs.  On the other hand, perhaps someone

will write a format conversion from format 1 to format 0 which might be

so easy to use in some setting that it would save you the trouble of

putting it into your program.

Programs which support several simultaneous tracks should be able to

save and read data in format 1, a vertically one-dimensional form, that

is, as a collection of tracks.  Programs which support several

independent patterns should be able to save and read data in format 2, a

horizontally one-dimensional form.  Providing these minimum capabilities

will ensure maximum interchangeability.

MIDI files can express tempo and time signature, and they have been

chosen to do so for transferring tempo maps from one device to another.

For a format 0 file, the tempo will be scattered through the track and

the tempo map reader should ignore the intervening events; for a format

1 file, the tempo map must (starting in 0.04) be stored as the first

track.  It is polite to a tempo map reader to offer your user the

ability to make a format 0 file with just the tempo, unless you can use

format 1.

All MIDI files should specify tempo and time signature.  If they don't,

the time signature is assumed to be 4/4, and the tempo 120 beats per

minute.  In format 0, these meta-events should occur at least at the

beginning of the single multi-channel track.  In format 1, these meta-

events should be contained in the first track.  In format 2, each of the

temporally independent patterns should contain at least initial time

signature and tempo information.

We may decide to define other format IDs to support other structures.  A

program reading an unfamiliar format ID should return an error to the

user rather than trying to read further.

3    Meta-Events

A few meta-events are defined herein.  It is not required for every

program to support every meta-event.  Meta-events initially defined

include:

FF 00 02 ssss   Sequence Number

This optional event, which must occur at the beginning of a track,

before any nonzero delta-times, and before any transmittable MIDI

events, specifies the number of a sequence.  The number in this track

corresponds to the sequence number in the new Cue message discussed at

the summer 1987 MMA meeting.  In a format 2 MIDI file, it is used to

identify each "pattern" so that a "song" sequence using the Cue message

to refer to the patterns.  If the ID numbers are omitted, the sequences'

locations in order in the file are used as defaults.  In a format 0 or 1

MIDI file, which only contain one sequence, this number should be

contained in the first (or only) track.  If transfer of several

multitrack sequences is required, this must be done as a group of format

1 files, each with a different sequence number.

FF 01 len text  Text Event

Any amount of text describing anything.  It is a good idea to put a text

event right at the beginning of a track, with the name of the track, a

description of its intended orchestration, and any other information

which the user wants to put there.  Text events may also occur at other

times in a track, to be used as lyrics, or descriptions of cue points.

The text in this event should be printable ASCII characters for maximum

interchange.  However, other character codes using the high-order bit

may be used for interchange of files between different programs on the

same computer which supports an extended character set.  Programs on a

computer which does not support non-ASCII characters should ignore those

characters.

(New for 0.06 ).  Meta event types 01 through 0F are reserved for

various types of text events, each of which meets the specification of

text events(above) but is used for a different purpose:

FF 02 len text  Copyright Notice

Contains a copyright notice as printable ASCII text.  The notice should

contain the characters (C), the year of the copyright, and the owner of

the copyright.  If several pieces of music are in the same MIDI file,

all of the copyright notices should be placed together in this event so

that it will be at the beginning of the file.  This event should be the

first event in the first track chunk, at time 0.

FF 03 len text  Sequence/Track Name

If in a format 0 track, or the first track in a format 1 file, the name

of the sequence.  Otherwise, the name of the track.

FF 04 len text  Instrument Name

A description of the type of instrumentation to be used in that track.

May be used with the MIDI Prefix meta-event to specify which MIDI

channel the description applies to, or the channel may be specified as

text in the event itself.

FF 05 len text  Lyric

A lyric to be sung.  Generally, each syllable will be a separate lyric

event which begins at the event's time.

FF 06 len text  Marker

Normally in a format 0 track, or the first track in a format 1 file.

The name of that point in the sequence, such as a rehearsal letter or

section name ("First Verse", etc.).

FF 07 len text  Cue Point

A description of something happening on a film or video screen or stage

at that point in the musical score ("Car crashes into house", "curtain

opens", "she slaps his face", etc.)

FF 2F 00        End of Track

This event is not optional.  It is included so that an exact ending

point may be specified for the track, so that it has an exact length,

which is necessary for tracks which are looped or concatenated.

FF 51 03 tttttt         Set Tempo, in microseconds per MIDI quarter-note

This event indicates a tempo change.  Another way of putting

"microseconds per quarter-note" is "24ths of a microsecond per MIDI

clock".  Representing tempos as time per beat instead of beat per time

allows absolutely exact long-term synchronization with a time-based sync

protocol such as SMPTE time code or MIDI time code.  This amount of

accuracy provided by this tempo resolution allows a four-minute piece at

120 beats per minute to be accurate within 500 usec at the end of the

piece.  Ideally, these events should only occur where MIDI clocks would

be located Q this convention is intended to guarantee, or at least

increase the likelihood, of compatibility with other synchronization

devices so that a time signature/tempo map stored in this format may

easily be transferred to another device.

FF 54 05 hr mn se fr ff SMPTE Offset  (New in 0.06 - SMPTE Format

specification)

This event, if present, designates the SMPTE time at which the track

chunk is supposed to start.  It should be present at the beginning of

the track, that is, before any nonzero delta-times, and before any

transmittable MIDI events.  The hour must be encoded with the SMPTE

format, just as it is in MIDI Time Code.  In a format 1 file, the SMPTE

Offset must be stored with the tempo map, and has no meaning in any of

the other tracks.  The ff field contains fractional frames, in 100ths of

a frame, even in SMPTE-based tracks which specify a different frame

subdivision for delta-times.

FF 58 04 nn dd cc bb    Time Signature

The time signature is expressed as four numbers.  nn and dd represent

the numerator and denominator of the time signature as it would be

notated.  The denominator is a negative power of two:  2 represents a

quarter-note, 3 represents an eighth-note, etc.  The cc parameter

expresses the number of MIDI clocks in a metronome click.  The bb

parameter expresses the number of notated 32nd-notes in a MIDI quarter-

note (24 MIDI Clocks).  This was added because there are already

multiple programs which allow the user to specify that what MIDI thinks

of as a quarter-note (24 clocks) is to be notated as, or related to in

terms of, something else.

Therefore, the complete event for 6/8 time, where the metronome clicks

every three eighth-notes, but there are 24 clocks per quarter-note, 72

to the bar, would be (in hex):

FF 58 04 06 03 24 08

That is, 6/8 time (8 is 2 to the 3rd power, so this is 06 03), 32 MIDI

clocks per dotted-quarter (24 hex!), and eight notated 32nd-notes per

MIDI quarter note.

FF 59 02 sf mi  Key Signature

sf = -7:  7 flats

sf = -1:  1 flat

sf = 0:  key of C

sf = 1:  1 sharp

sf = 7: 7 sharps

mi = 0:  major key

mi = 1:  minor key

FF 7F len data  Sequencer-Specific Meta-Event

Special requirements for particular sequencers may use this

event type:  the first byte or bytes of data is a manufacturer ID.

However, as this is an interchange format, growth of the spec proper is

preferred to use of this event type.  This type of event may be used by

a sequencer which elects to use this as its only file format;

sequencers with their established feature-specific formats should

probably stick to the standard features when using this format.

4   Program Fragments and Example MIDI Files

Here are some of the routines to read and write variable-length numbers

in MIDI Files.  These routines are in C, and use getc and putc, which

read and write single 8-bit characters from/to the files infile and

outfile.

WriteVarLen (value)

register long value;

{

register long buffer;

buffer = value & 0x7f;

while ((value >>= 7) > 0)

{

buffer <<= 8;

buffer |= 0x80;

buffer += (value & 0x7f);

}

while (TRUE)

{

putc(buffer,outfile);

if (buffer & 0x80)

buffer >>= 8;

else

break;

}

}

doubleword ReadVarLen ()

{

register doubleword value;

register byte c;

if ((value = getc(infile)) & 0x80)

{

value &= 0x7f;

do

{

value = (value << 7) + ((c = getc(infile)) & 0x7f);

} while (c & 0x80);

}

return (value);

}

As an example, MIDI Files for the following excerpt are shown below.

First, a format 0 file is shown, with all information intermingled;

then, a format 1 file is shown with all data separated into four tracks:

one for tempo and time signature, and three for the notes.  A resolution

of 96 "ticks" per quarter note is used.  A time signature of 4/4 and a

tempo of 120, though implied, are explicitly stated.

The contents of the MIDI stream represented by this example are broken

down here:

Delta Time(decimal)  Event Code (hex)   Other Bytes (decimal)

Comment

0       FF 58   04 04 02 24 08  4 bytes: 4/4 time, 24 MIDI

clocks/click,

8 32nd notes/24 MIDI clocks

0       FF 51   03 500000       3 bytes: 500,000 5sec per quarter-note

0       C0      5       Ch. 1, Program Change 5

0       C0      5       Ch. 1, Program Change 5

0       C1      46      Ch. 2, Program Change 46

0       C2      70      Ch. 3, Program Change 70

0       92      48  96  Ch. 3 Note On C2, forte

0       92      60  96  Ch. 3 Note On C3, forte

96      91      67  64  Ch. 2 Note On G3, mezzo-forte

96      90      76  32  Ch. 1 Note On E4, piano

192     82      48  64  Ch. 3 Note Off C2, standard

0       82      60  64  Ch. 3 Note Off C3, standard

0       81      67  64  Ch. 2 Note Off G3, standard

0       80      76  64  Ch. 1 Note Off E4, standard

0       FF 2F   00      Track End

The entire format 0 MIDI file contents in hex follow.  First, the header

chunk:

4D 54 68 64     MThd

00 00 00 06     chunk length

00 00   format 0

00 01   one track

00 60   96 per quarter-note

Then, the track chunk.  Its header, followed by the events (notice that

running status is used in places):

4D 54 72 6B     MTrk

00 00 00 3B     chunk length (59)

Delta-time      Event   Comments

00      FF 58 04 04 02 18 08    time signature

00      FF 51 03 07 A1 20       tempo

00      C0 05

00      C1 2E

00      C2 46

00      92 30 60

00      3C 60   running status

60      91 43 40

60      90 4C 20

81 40   82 30 40        two-byte delta-time

00      3C 40   running status

00      81 43 40

00      80 4C 40

00      FF 2F 00        end of track

A format 1 representation of the file is slightly different.  Its header

chunk:

4D 54 68 64     MThd

00 00 00 06     chunk length

00 01   format 1

00 04   four tracks

00 60   96 per quarter-note

First, the track chunk for the time signature/tempo track.  Its header,

followed by the events:

4D 54 72 6B     MTrk

00 00 00 14     chunk length (20)

Delta-time      Event   Comments

00      FF 58 04 04 02 18 08    time signature

00      FF 51 03 07 A1 20       tempo

83 00   FF 2F 00        end of track

Then, the track chunk for the first music track.  The MIDI convention

for note on/off running status is used in this example:

4D 54 72 6B     MTrk

00 00 00 10     chunk length (16)

Delta-time      Event   Comments

00      C0 05

81 40   90 4C 20

81 40   4C 00   Running status: note on, vel = 0

00      FF 2F 00        end of track

Then, the track chunk for the second music track:

4D 54 72 6B     MTrk

00 00 00 0F     chunk length (15)

Delta-time      Event   Comments

00      C1 2E

60      91 43 40

82 20   43 00   running status

00      FF 2F 00        end of track

Then, the track chunk for the third music track:

4D 54 72 6B     MTrk

00 00 00 15     chunk length (21)

Delta-time      Event   Comments

00      C2 46

00      92 30 60

00      3C 60   running status

83 00   30 00   two-byte delta-time, running status

00      3C 00   running status

00      FF 2F 00        end of track

5   MIDI Transmission of MIDI Files

Since it is inconvenient to exchange disks between different computers,

and since many computers which will use this format will have a MIDI

interface anyway, MIDI seems like a perfect way to send these files from

one computer to another.  And, while we're going through all the trouble

to make a way of sending MIDI Files, it would be nice if they could send

any files (like sampled sound files, text files, etc.)

Goals

The transmission protocol for MIDI files should be reasonably efficient,

should support fast transmission for computers which are capable of it,

and slower transmission for less powerful ones.  It should not be

impossible to convert a MIDI File to or from an arbitrary internal

representation on the fly as it is transmitted, but, as long as it is

not too difficult, it is very desirable to use a generic method so that

any file type could be accommodated.

To make the protocol efficient, the MIDI transmission of these files

will take groups of seven 8-bit bytes and transmit them as eight 7-bit

MIDI data bytes.  This is certainly in the spirit of the rest of this

format (keep it small, because it's not that hard to do).  To

accommodate a wide range of transmission speeds, files will be

transmitted in packets with acknowledge -- this allows data to be stored

to disk as it is received.  If the sender does not receive a response

from a reader in a certain amount of time, it can assume an open-loop

situation, and then just continue.

The last edition of MIDI Files contained a specialized protocol for

sending just MIDI Files.  To meet a deadline, unfortunately I don't have

time right now to propose a new generalized protocol.  This will be done

within the next couple of months.  I would welcome any proposals anyone

else has, and would direct your attention to the proposal from Ralph

Muha of Kurzweil, available in a recent MMA bulletin, and also directly

from him.

--

+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+

|  Wity Ganda                    |  wity@iastate.edu                    |

|  Iowa State University         |  as101@cleveland.freenet.edu         |

+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+=-+

############################################################################################