ICECOLOR v1.1 programmed by Leviathan of iCE

General Description

-------------------

iCECOLOR disables the blink on your monitor. What does this mean? This

means that you get 16 colors in the background as well as 16 colors in the

foreground.

How it works

------------

The attribute byte is set up like this :

before :                  after :

|7|6|5|4|3|2|1|0|          |7|6|5|4|3|2|1|0|

| |   | |     |            |     | |     |

| +---+ +-----+            +-----+ +-----+

|   |      |                  |       |

blink back  foreground      background foreground

Usesuly when you change video modes everything goes back to normal, but I've

taken over interrupt 10H so it turns off the blink everytime you change modes

Installation

------------

There is no way to un-install ICECOLOR.COM from memory, but it only takes up

400 bytes so I don't think it will hurt you.

Using ICECOLOR with The Draw

----------------------------

The draw has no idea that ICECOLOR is installed, so it still thinks it is

using the setup of : 8 background, 16 forefround, 16 blinking. You will

noctice that the blinking colors no longer blink, and their background is

high intensity black. This is because black w/o blink = dark grey. so you

select you colors as follows :

regular background colors work normaly :

0-7 for background and 0-15 for foreground

for high intensity background colors :

select from colors 16-31 to choose your foreground color, while

telling the computer that you whant a high-intensity background

then choose colors 0-7 for you background color, but they will

now be high intensity

problems w/ the draw :

When you are using high intensity colors and you use the delete key,

or clear the screen, it will use high intensity black for the background,

so make shure that you are in a normal background color when you delete

or clear the screen.

Source code availibility

------------------------

ICECOLOR was written in 100% assembly. If you would like the source, or have

a question, try to get in touch w/ me... somehow.