Bells, Whistles, and Sound Boards

PRO AUDIO SPECTRUM FAMILY TECHNICAL REFERENCE

Copyright (c) 1993-97, Edward Schlunder

See COPYING.ES for redistribution details

Contents:

1.      Introduction

1.1     I/O Address Map

1.2     MVSOUND.SYS

Part 1. Introduction

����������������������������������������������������������������������������

This information is from personal experimentation, information

gathered by other people, and by examining other people's source

code. It was gathered from many sources and therefore may contain

errors or omissions. I do not claim that any of this is "official"

in any way. If you find anything incorrect, please help others by

emailing to correction(s) to zilym@hndymn.stat.com.

1.1 I/O Address Map

Possible Pro AudioSpectrum Configurations:           [] - Factory Default

Base I/O: [388h], 384h, 38Ch, 388h

IRQ:      2, 3, [5], 7, 10, 11, 12, 15

DMA:      0, [3], 5, 6, 7 (DMA 1 is used for hardware SB emulation)

-����������������������������������������������������������������������-

� I/O Address �  Description/Usage                   � Read/Write    �

-����������������������������������������������������������������������-

B88h           Audio Mixer Control                  � Read/Write �

B8Ah           Audio Filter Control                 � Read/Write �

7 6 5 4 3 2 1 0

Sample Counter Enable � � � ���������

Sample Rate Enable � � � Filter Select:

Output Mute(0)/Enable(1) �   00000 - None [default]

00001 - 17897 Hz

00010 - 15909 Hz

01001 - 11931 Hz

10001 - 8948 Hz

11001 - 5965 Hz

00100 - 2982 Hz

B89h           Interrupt Control Status             � Read/Write �

B8Bh           Interrupt Control                    � Write      �

7 6 5 4 3 2 1 0

Board Revision Number ����� � � � � � Left FM IRQ Enable

MIDI IRQ Enable � � � � Right FM IRQ Enable

� � Sample Rate Timer enable

� Sample Count timer enable

B8Bh           Interrupt Control read back          � Read       �

F88h           PCM Direct Data I/O LSB              � Read/Write �

F89h           PCM Direct Data I/O MSB (for 16bit)  � Read/Write �

F8Ah           Cross Channel Control                � Read/Write �

7 6 5 4 3 2 1 0

DRQ Hook Enable � � � � � � � � Connect RIN->ROUT

PCM State Machine Enable � � � � � � Connect LIN->ROUT

Mono(1)/Stereo(0) Output � � � � Connect RIN->LOUT

DAC(1)/ADC(0) � � Connect LIN->LOUT

1388h          Sample Rate Timer (t0)               � Read/Write �

1389h          Sample Byte Count                    � Read/Write �

138Bh          Local Timer Control                  � Read/Write �

7 6 5 4 3 2 1 0

Timer Select ��� ��� ����� � BCD (1)/Binary (0)

Read/Write �   � Timer Mode

8388h          System Config 1                      � Read/Write �

8389h          Sample Configuration                 � Read/Write �

7 6 5 4 3 2 1 0

VCO Locked (Rate Clock Valid) � ��� � � � ��� Oversample 1x, 2x, 4x

Slave Port Bits �   � � � 16bit(1)/8bit(0)

Signed(1)/Unsigned(0) � � 12bit Interleaving (b2 set)

838Ah          System Config 3                      � Read/Write �

838Bh          System Config 4                      � Read/Write �

������ MVSOUND.SYS ����������������������������                   5.4.2.0

������������������������������������������������������������������������

Function:     0, Int 2Fh

Description:  Verifies that MVSOUND.SYS is loaded.

Calling:      ax = BC00h

bx = '??'

cx = 0

dx = 0

Returns:      xor bx, cx

xor bx, dx

bx = 'MV' if MVSOUND.SYS is installed

������������������������������������������������������������������������

Function:     1, Int 2Fh

Description:  Get MVSOUND.SYS Version

Calling:      ax = BC01h

bx = 'm '

cx = ' v'

dx = '  '

Returns:      bx      ASCII major version

cx      ASCII minor version

������������������������������������������������������������������������

Function:     2, Int 2Fh

Description:  Get Pro AudioSpectrum State Table Address

Calling:      ax = BC02h

Returns:      bx:dx   PAS state table address

������������������������������������������������������������������������

Function:     3, Int 2Fh

Description:  Get Pro AudioSpectrum State Table Address

Calling:      ax = BC02h

Returns:      bx:dx   PAS state table address